Power

- A 60 kg box is lifted a distance of 10 meters straight up at a constant speed using a rope.  How much average power is required to complete this task in 5 seconds?

- Harry and Bob are tow workers being considered for a job at the UPS loading dock.  Harry boasts that he can lift a 100 kg box 2 meters vertically, in 3 seconds.  Bob counters with his claim of lifting a 200 kg box, 5 meters vertically, in 20 seconds.  Which worker has the greater average power rating?

- A 1994 Mustang is driving down a road with a constant speed f 30 m/s.  The engine must exert a 5000N force (on average) to maintain that speed.

- What is the power rating of the engine?

- How does this compare to a 220 horsepower dodge stealth? (1 hp = 746 W)

- An 82 kg hiker climbs Mt. Humphrey.  During a two hour period, the hiker’s vertical elevation increases by 540 meters.

- Calculate the climber’s change in gravitational potential energy.

- Find the average power generated during this increase in gravitational potential energy.

- How long would it take a 7.5 kW motor to raise a 500 kg piano to an apartment window 10 meters above the ground?

- A fire truck requires a pump that can move 10 kg of water vertically 10 meters each second.  Does a 1.25 hp pump have enough power to do the job?  Why or why not?  If not, what would be necessary?